SPECIFICATION

of work to be done and materials to be used in carrying

out the works shown on the accompanying drawings

~

(project name)

~

(project address)

~

(owners name)

Job Number:	~

Date:	~

# 4234P	PALLISIDE UPVC WEATHERBOARD CLADDING

Masterspec sections must be customised to suit the project being specified, by removing irrelevant information and adding project-specific information and selections.

## 1.	GENERAL

This section relates to the supply and fixing of Palliside uPVC weatherboards.

Modify or extend the above description to suit the project being specified.

Related work

### 1.1	RELATED SECTIONS

Refer to ~ for ~.

Include cross references to other sections where these contain related work.  This may involve Building Wraps or Rigid Air Barriers where specified separately.

Documents

### 1.2	DOCUMENTS REFERRED TO

Documents referred to in this section are:

NZBC E2/AS1	External moisture

AS/NZS 1170.2	Structural design actions - Wind actions

NZS 3602	Timber and wood-based products for use in building

NZS 3604	Timber framed buildings

Documents listed above and cited in the clauses that follow are part of this specification.  However, this specification takes precedence in the event of it being at variance with the cited document.

Delete from the DOCUMENTS clause any document not cited.  List any additional cited documents.

RELATED DOCUMENTS:

Refer to the following related documents when preparing this section:

BRANZ BU 353	Ground clearances

BRANZ BU 407	Walls on exposed sites

BRANZ BU 439	Condensation risk in walls

BRANZ BU 449	Keeping water out - Timber-framed walls

BRANZ BU 453	Fasteners selection

BRANZ BU 460	Internal moisture control

BRANZ BU 467	Principles of flashing design

BRANZ publication	Selecting wall claddings

### 1.3	MANUFACTURER'S DOCUMENTS

Palliside documents relating to work in this section are:

Palliside	Technical guide

Palliside	Direct fix installation guide

Palliside	Drained cavity installation guide

BRANZ Appraisal 490 - Palliside Cellular uPVC Weatherboard Direct Fixed Cladding System

BRANZ Appraisal 491 - Palliside Cellular uPVC Weatherboard Cavity System

Copies of the above literature are available at

Web:	www.palliside.co.nz

Telephone:	0800 439 639

It is important to ensure that all personnel on site have access to accurate, up to date technical information on the many products, materials and equipment used on a project.  In most cases individual products are not used in isolation, but form part of a building process.  Also a particular manufacturer's and/or supplier's requirements for handling, storage, preparation, installation, finishing and protection of their product can vary from what might be considered the norm.  Access to technical information can help overcome this potential problem.

Requirements

### 1.4	NO SUBSTITUTIONS

Substitutions are not permitted to any specified system, or associated components and products.

### 1.5	QUALIFICATIONS

Work to be carried out by tradesmen experienced, competent and familiar with the materials and techniques specified.

### 1.6	WARRANTY - MANUFACTURER/SUPPLIER

Provide a material manufacturers warranty:

25 years:	For defects in the material

-	Provide this warranty on the manufacturer's standard form.

-	Commence the warranty from the date of purchase.

Refer to the section WARRANTIES for additional requirements.

Modify or expand the clause to suit project requirements, options include:

-	Commence the warranty from the date of completion of this part of the contract work

Performance

Note that in using the next clause you are setting a standard of performance for the contractor to meet.  Specifiers should therefore consider adding some means of confirming (say by testing or certification) that the performance criteria will be or have been met.

### 1.7	FIXINGS, WIND

Design and use the fixings appropriate for the wind zone (R) and topographical classification (T) of this site and building height; as required by Palliside technical and installation guides and NZS 3604, and the wind loads on various wall areas as given by AS/NZS 1170.

Modify this clause depending on whether NZS 3604 or AS/NZS 1170 are being used as a design basis for the project.  Note: Palliside has been BRANZ appraised as being within the scope of NZS 3604 Wind Zones up to and including "Very High".

## 2.	PRODUCTS

Materials

### 2.1	BUILDING WRAP

Waterproof, breather type to NZBC E2/AS1, Table 23: Properties of roof underlays and building wraps.

Delete this clause if specified separately.  Modify this clause depending on whether a building wrap or a rigid air barrier is being used.

### 2.2	EXTERIOR CAVITY BATTENS

Radiata pine battens, minimum 45mm wide x 18mm thick, H3.1 treated, height to match timber framing studs.  To NZS 3602, Table 1, reference 1D.10, Requirements for wood-based building components to achieve a 50-year durability performance.

Delete this clause if specified separately or not required.  Modify this clause depending on whether timber or proprietary battens are being used.

### 2.3	EXTERIOR CAVITY VERMIN-PROOFING

Palliside vermin tray to NZBC E2/AS1: clause 9.1.8.3 and figure 66.

### 2.4	PALLISIDE WEATHERBOARDS

Palliside uPVC weatherboards, 6mm thick, manufactured with a cellular core and a co-extruded outer layer

Refer to Palliside Technical Guide for further details.

Components

### 2.5	TRIMS AND JOINTERS

Palliside starting trims, jointers, corner soakers, flashings and finishing trims, colour matched to the Palliside weatherboards where exposed.

### 2.6	FASTENERS

Fasteners to minimum durability requirements of the NZBC.  Refer to NZS 3604, section 4 Durability, for requirements for fixing's material to be used in relation to the exposure conditions.  Refer to Palliside, direct fix or drained cavity, installation guides for suitable nails, impulse nails and screws.

Accessories

### 2.7	SEALANT

MS sealant colour matched to weatherboards.  Refer to Palliside installation guides for application requirements.

## 3.	EXECUTION

Conditions

### 3.1	STORAGE

Take delivery of the Palliside weatherboards and lay horizontally on a smooth level surface.  Protect edges and corners from damage and cover to keep dry until fixed.

### 3.2	HANDLING

Avoid distortion and contact with potentially damaging surfaces.  Use 2 workers to carry and install the Palliside weatherboards.  Do not drag weatherboards across each other, or across other materials.

### 3.3	SUBSTRATE

Do not commence work until the substrate is of the required standard for the specified finish; plumb, level and in true alignment.  Moisture content of timber framing must not exceed the requirements specified by NZS 3602 to minimise shrinkage and movement after sheets are fixed.

### 3.4	TEMPERATURE

Installation to be carried out within a temperature range of 10°C to 25°C.  Out side of this range special care must be taken, refer to Palliside installation guides.

Application - generally

### 3.5	FIX BUILDING WRAP

Run and fix building wrap in full height rolls to wall framing, with fixing and end laps to NZS 3604 and the wrap manufacturer's requirements and with the wrap not damaged in any way.

Delete this clause if specified separately.  Modify this clause depending on whether a building wrap or a rigid air barrier is being used.

### 3.6	INSTALL CAVITY BATTENS

Install 18mm minimum thick cavity battens to NZBC E2/AS1: 9.0 Wall claddings, where required.  Fix vertical cavity battens to wall framing studs.  The battens are fixed by the cladding fixings which will penetrate the wall framing studs, through the building wrap or rigid air barrier.  Seal the top of the cavity and install vermin-proofing at base.  Do not use horizontal cavity battens.  Use cavity spacers where fixing is required between cavity battens.

Note that it is important that the openings in the vermin proofing are kept clean and unobstructed in order to maintain drainage and venting of the cavity.

### 3.7	PENETRATIONS AND FLASHINGS

Confirm that exterior wall openings have been prepared ready for the installation of all window and door frames and other penetrations through the cladding.  Required preparatory work includes the following:

-	Building wrap appropriately incorporated with penetration and junction flashings.

-	Materials lapped in a way that water tracks down to the exterior face of the building wrap.

-	Wall cladding underlay/building wrap to openings finished and dressed off ready for the installation of window and door frames and other penetrations

-	Claddings neatly finished off to all sides of openings

-	Installation of flashings (those required to be installed prior to installation of penetrating elements).

Modify this clause depending on whether a building wrap or a rigid air barrier is being used.

Refer to Palliside technical and installation guides for information on window details.  Also refer to the Windows Association of New Zealand website (www.wanz.org.nz) for information on the WANZ WIS Window Installation System.  This covers the WANZ recommendations on the preparation of window/door openings, minimum clearances between rough openings and the window/door frame, dressing of the wall wrap into the prepared opening, application of flexible flashing tape to the sill and top corners of the opening, installation of window/door frames and flashings, sealing of the window/door frame into the opening to create a pressure equalisation cavity, installation of flashings and the maintenance of appropriate clearances between the frame and the surrounding construction.

### 3.8	INSTALL PALLISIDE BASE ACCESSORIES

Install Palliside starter strips, vermin trays and channels to Palliside installation guides.

### 3.9	INSTALL PALLISIDE WEATHERBOARDS

Cut Palliside weatherboards to required lengths and fit joints off-stud using Palliside moulded soakers.

### 3.10	INSTALL FLASHINGS

Install flashings at all wall openings, penetrations, junctions, connections, window sills, heads and jambs to Palliside technical and installation guides and to NZBC E2/AS1.

### 3.11	INSTALL PALLISIDE EXTERNAL CORNER SOAKERS

Install the Palliside corner soaker and soaker cap to Palliside technical and installation guides.  Soaker cap profile to match the weatherboard profile, and be aligned accurately.

### 3.12	INSTALL PALLISIDE BOXED AND INTERNAL CORNERS

Install the Palliside base piece and male cap piece to Palliside technical and installation guides.  Install Palliside moulded end plugs to Palliside technical and installation guides.

Modify this clause for non-standard corners or timber boxed corners.

### 3.13	SEALANTS AND AIR SEALS

Install air seals to requirements of Palliside technical guides and to NZBC E2/AS1 clause 9.1.6.  Flash pipe penetrations and add pipe flanges and sealant where required to provide additional protection.  Complete seal to electrical meter box to Palliside design details.  Seal all other junction to Palliside design details.

Completion

### 3.14	REPLACE

Replace all damaged or marked elements.

### 3.15	LEAVE

Leave work to the standard required for following procedures.

### 3.16	REMOVE

Remove debris, unused materials and elements from the site.

## 4.	SELECTIONS

### 4.1	BUILDING WRAP

Brand/type:	~

Flashing tape:	~

Delete this clause if specified separately.  Modify this clause depending on whether a building wrap or a rigid air barrier is being used.  Note: if wrap is in direct contact with the Palliside weatherboard, it must be an absorbent type.  Check with wrap manufacturer.

### 4.2	CAVITY BATTENS

Timber species:	Radiata pine

Treatment:	H3.1

Delete this clause if specified separately or not required.  Modify this clause depending on whether timber or proprietary battens are being used.

### 4.3	PALLISIDE WEATHERBOARDS

Brand/type:	Palliside uPVC Weatherboard

Profile/texture:	~

Colour:	~

Profile texture options: Rusticated smooth, Rusticated woodgrain, Traditional smooth

Colour options:	White, Calico, Sandstone, Slate, Tea, Riverstone

### 4.4	JOINTERS

Type:	Palliside moulded flat soaker to match weatherboard profile

Type option:	2 part jointer with moulded end plugs

### 4.5	CORNER SOAKERS

Type:	Palliside moulded corner soaker to match weatherboard profile

### 4.6	BOXED CORNERS

Type:	Palliside ~

End plugs:	Palliside moulded end plugs to match weatherboard profile.

Type options:	internal 90° corner, external 90° corner, 135° corner

Modify for non-standard corners.

### 4.7	FLASHINGS

Type:	Palliside proprietary flashings

### 4.8	FIXINGS FOR PALLISIDE ITEMS

Concealed Fixing:	~

Exposed Fixing:	~

Fixings:	Type, finish and size to Palliside installation guides.

For some higher exposures concealed and exposed fixings may be different.