User Note: Select the applicable access requirement in Section 1.7 and remove all irrelevant products and clauses. Alternatively, request a project-specific specification through Masterspec. Contact Total Height Safety for a fit-for-purpose access design.

7938TH - TOTAL HEIGHT SAFETY FALL PROTECTION AND ACCESS SYSTEMS

1	GENERAL

This section relates to the manufacture, supply and installation of Total Height Safety permanent fall protection and access systems required for routine maintenance and servicing activities on buildings.

It includes:

- Anchor systems

- Static line systems

- Ridged rail systems

- Davits and Needles

- Ladder systems

- Walkway

Guardrail systems

- Stairs

- Harnesses & Accessories

1	Documents

1.1	DOCUMENTS

Refer to the general section 1233 REFERENCED DOCUMENTS.  The following documents are specifically referred to in this section:

| AS-1657-2018 | Platforms-Walkways-Stairways-Ladders |
| --- | --- |
| AS/NZS 1891.2 | Industrial fall-arrest systems and devices - Horizontal lifeline and rail systems |
| AS/NZS 1891.4 | Industrial fall-arrest systems and devices - Selection, use and maintenance |
| AS/NZS 4488.1 | Industrial rope access systems - Specifications |
| AS/NZS 4488.2 | Industrial rope access systems - Selection, use and maintenance |
| AS/NZS 5532 | Manufacturing requirements for single point anchor device used for harness based work at height |
| NZQA 23229 | Use safety harness system when working at height. |
| AS/NZS 5532 | Manufacturing requirements for single-point anchor device used for harness-based work at height |
| WorkSafe | Guidelines for the provision of facilities and general safety in the construction industry |
| WorkSafe | Best practice guidelines for working at height in NZ – Good Practice Guidelines |
| WorkSafe | Working on roofs - Good Practice Guidelines |

1.2	MANUFACTURER/SUPPLIER DOCUMENTS

Manufacturer's documents relating to this part of the work:

- SafetyLink® Pty Limited Access Systems - Performance and 10 Year Corrosion Resistance Guarantee

- Owner's Operations Manual

- Shop Drawings Roof Plan Detailing Access System

- Compliance Certificate to AS/NZS 1891.4

Supplier contact details

| Company: | Total Height Safety |
| --- | --- |
| Web: | www.totalheightsafety.co.nz |
| Email: | tenders@totalheightsafety.co.nz |
| Telephone: | 0508 443 444 |

Warranties

1.3	WARRANTY - MANUFACTURER/SUPPLIER

Provide a material manufacturer/supplier warranty:

| 10 years | Warranty period for materials, refer to Total Height Safety for terms and conditions. |
| --- | --- |

Provide these warranties on the Total Height Safety standard forms; (if not available use the standard form in the general section 1237WA WARRANTY AGREEMENT)

Commence the warranty from the date of Installation of the contract works.

Refer to the general section 1237 WARRANTIES for additional requirements.

14	WARRANTY - INSTALLER

Provide an installer warranty:

| 5 years | For installation |
| --- | --- |

Provide this warranty on the installer standard form; (if not available use the standard form in the general section 1237WA WARRANTY AGREEMENT)

Commence the warranty from the date of Installation of the contract works.

Refer to the general section 1237 WARRANTIES for additional requirements.

1.5	QUALIFICATIONS WORKERS – MANUFACTURER / SUPPLIER REQUIREMENTS

Installation to be carried out by Total Height Safety certified installers.  Refer to 1270 CONSTRUCTION for additional requirements relating to qualifications.

1.6	NO SUBSTITUTIONS OR CHANGES TO SYSTEM DESIGN

Substitutions are not permitted to any Total Height Safety specified system, or associated components and products listed in this section, nor is it permitted to change the access design.

1.7	SHOP DRAWINGS

Refer to the general section 1235 SHOP DRAWINGS for the requirements for submission and review and the provision of final shop drawings.

Provide shop drawings to show the general arrangement including, but not limited to:

User Restraint System:

- Location and type of user Restraint System

- Extent of access with 2m restraint lanyard

User Fall Arrest System:

- Location and type of user Fall Arrest System

User Fall Arrest and Abseil System:

- Location and type of Fall Arrest and Abseil System

- Location of system appropriate for building exterior maintenance

1.8	 AS-BUILT DOCUMENTS

Refer to the general section 1238 AS-BUILT DOCUMENTATION for the requirements for submission and review of as-built documents and records.

Provide the following as-built documents and records:

User Access System:

- As built location and type of Access System

Provide final as-built information prior to the certified Practical Completion date.

1.9	INFORMATION FOR OPERATION AND MAINTENANCE

Refer to the general section 1239 OPERATION & MAINTENANCE for provision of the following general operation and maintenance information as electronic PDF format documents, to be supplied by the contractor:

- Total Height Safety System Roof Care and Maintenance Plan

- Owner's Operations Manual

- As-built roof plan detailing access system

- Compliance Certificate to AS/NZS 1891.4

- Quality Assurance form

Compliance information

1.10	INFORMATION REQUIRED FOR CODE COMPLIANCE

Provide the following compliance documentation:

- Manufacturer's warranty

- Installer's warranty

- Installer's compliance certificate

- Producer Statement PS1 Design - from  a suitably qualified person

- Producer Statement PS3 Construction - from the installer

- Producer Statement PS4 Construction Review - from a suitably qualified person

Quality control and assurance

1.11	INSPECTIONS

Total Height Safety are required to carry out inspections outside their normal activities and provide reports or other documents in support of the work carried out (eg PS1).

Total Height Safety are required to carry out or engage consultants to carry out inspections of parts of the work.  It will typically be expected that the person carrying out the inspections will be required to provide Producer Statements in support of the work carried out (eg PS4).

2	PRODUCTS

2.1	Materials - anchor point systems

SAFETYLINK FALL-ARREST ANCHOR POINT SYSTEMS

SafetyLink Anchor Point System manufactured to AS/NZS 5532, complies with AS/NZS 1891.4  and AS/NZS 4488.2.

Systems include:

- SafetyLink® FrogLink®

- SafetyLink® SurfaceLink®

- SafetyLink® RetroLink®

2.2	SAFETYLINK ABSEIL ANCHOR POINT SYSTEMS

SafetyLink Anchor Point System manufactured to AS/NZS 5532, complies with AS/NZS 1891.4  and AS/NZS 4488.2.

Systems include:

- SafetyLink® SurfaceLink®

- SafetyLink® ConcreteLink®

- SafetyLink® TuffPost®

Cable Lifeline System

2.2 	SAFETYLINK STATIC CABLE LINE SYSTEM

SafetyLink® FrogLine System, energy-absorbing horizontal lifeline manufactured from 316 Stainless Steel and marine-grade aluminium, to AS/NZS 1891.2 and AS/NZS 1891.4.  Refer to SELECTIONS.

2.3	SAFETYLINK LADDER VERTICAL CABLE SYSTEM

SafetyLink® V-Line Vertical Cable System, energy-absorbing vertical cable shuttle/traveller manufactured from 316 Stainless Steel to AS/NZS 1891.2 and AS/NZS 1891.4.  Refer to SELECTIONS.

Rail Systems
2.4	SAFETYLINK X-RAIL ABSEIL RAIL SYSTEM

SafetyLink® X-Rail System, abseil rail system manufactured from marine grade Aluminium to AS/NZS 5532, AS/NZS 1891.4 and AS/NZS 4488.2, for wall, floor and overhead installation.  Refer to SELECTIONS.

Davit Systems

2.5	SAFETYLINK DAVIT SYSTEM

SafetyLink® Davit System, abseil system manufactured from marine grade Aluminium to AS/NZS 5532, AS/NZS 1891.4 and AS/NZS 4488.2, for wall and floor installation.  Refer to SELECTIONS.

Stair, Walkway, Guardrail and Stair Systems

2.6	SAFETYLINK FASTFIT WALKWAY SYSTEM

SafetyLink® FastFit Walkway, roof-mountable walkway system manufactured from marine-grade aluminium to AS 1657.  Refer to SELECTIONS.

2.7	SAFETYLINK FASTFIT LADDER SYSTEM

SafetyLink® FastFit Ladder, permanent fixed Ladder system manufactured from marine-grade aluminium to AS 1657 available caged and uncaged.  Refer to SELECTIONS.

2.8	SAFETYLINK FASTFIT GUARDRAIL SYSTEM

SafetyLink® FastFit Guardrail roof/structure mountable guardrail system, manufactured from marine-grade aluminium to AS 1657.  Refer to SELECTIONS.

2.9	SAFETYLINK FASTFIT HATCH GUARDRAIL ENCLOSURE

SafetyLink® FastFit Hatch Guardrail Enclosure, roof mountable enclosure manufactured from marine grade Aluminium to AS 1657, available with self-close gate.  Refer to SELECTIONS.

2.10	SAFETYLINK LADDERLINK LADDER STABILISING DOCK

SafetyLink® LadderLink Ladder Brackets manufactured from marine grade aluminium, available in a range of mounting options with two tie-off points.  Refer to SELECTIONS.

2.11	SAFETYLINK MODULAR STAIR SYSTEM

SafetyLink® Modular Stair systems, manufactured from marine-grade aluminium to AS 1657.  Refer to SELECTIONS.

Harness Gear & Accessories

2.12	SAFETYLINK® ROOF WORKERS KIT (2 Sets)

SafetyLink® Roof Workers Kit, system-appropriate Personal Protection Equipment (PPE) to AS/NZS 4488.1 and WorkSafe Best practice guidelines for working at height in NZ - Good Practice Guidelines.

3	EXECUTION

Installation

3.1	INSTALLATION

Install Total Height Safety access systems in accordance with Producer Statement - Design (PS1), shop drawings, AS/NZS 1891.2 and AS/NZS 1891.4.

Completion & Commissioning

3.2	COMPLETION - TESTS & CERTIFICATION

Refer to 1270 CONSTRUCTION for general test and certification requirements at completion. Inspect and test the installation to AS/NZS 1891.2 and AS/NZS 1891.4.  On completion, provide installer’s Producer Statement - Construction (PS3) and a Producer Statement - Construction Review (PS4) from a suitably qualified person.

4.	SELECTIONS

For further details on selections go to www.totalheightsafety.co.nz.

Substitutions are not permitted for the following Total Height Safety systems.

Materials - Fall-arrest anchor point systems

4.1	SAFETYLINK FROGLINK ANCHOR POINT SYSTEM

| Supplier: | Total Height Safety |
| --- | --- |
| Type/Brand: | SafetyLink FrogLink |
| Use type: | Restraint / Fall Arrest |

4.2	SAFETYLINK SURFACELINK ABSEIL ANCHOR POINT SYSTEM

| Supplier: | Total Height Safety |
| --- | --- |
| Type/Brand: | SafetyLink SurfaceLink Abseil |
| Use type: | Rope Positioning / Abseil |

4.3	SAFETYLINK RETROLINK ANCHOR POINT SYSTEM

| Supplier: | Total Height Safety |
| --- | --- |
| Type/Brand: | SafetyLink RetroLink |
| Use type: | Fall Arrest / Rope Positioning / Abseil |

4.4	SAFETYLINK CONCRETELINK ANCHOR POINT SYSTEM

| Supplier: | Total Height Safety |
| --- | --- |
| Type/Brand: | SafetyLink ConcreteLink |
| Use type: | Fall Arrest / Abseil |

System Posts

4.5	SAFETYLINK TUFF-POST ANCHOR

| Supplier: | Total Height Safety |
| --- | --- |
| Type/Brand: | SafetyLink Tuff-Post Anchors |
| Use type: | System mounting post |
| System Use: | SafetyLink Anchors and FrogLine |
| Mounting substrate: | Concrete warm roof system |

Cable Lifeline Systems

4.6	SAFETYLINK FROGLINE STATIC LINE SYSTEM

| Supplier: | Total Height Safety |
| --- | --- |
| Type/Brand: | SafetyLink FrogLine |
| Use type: | Restraint / Fall Arrest |

4.7	SAFETYLINK V-LINE VERTICAL CABLE SYSTEM

| Supplier: | Total Height Safety |
| --- | --- |
| Type/Brand: | SafetyLink V-Line |
| Use type: | Fall Arrest |
| Mounting substrate: | Fixed Ladder |

Rail Systems

4.8	SAFETYLINK X-RAIL SYSTEM

| Supplier: | Total Height Safety |
| --- | --- |
| Type/Brand: | SafetyLink X-Rail Rail System |
| Use type: | Fall Arrest / Rope Positioning / Abseil |

4.9	SAFETYLINK ULTRA-RAIL WIDE SPAN RAIL SYSTEM

| Supplier: | Total Height Safety |
| --- | --- |
| Type/Brand: | SafetyLink Ultra-Rail Rail System |
| Use type: | Abseil |

Ladder / Walkway / Guardrail / Stair Systems

4.10	SAFETYLINK LADDERLINK LADDER STABILISING DOCK

| Supplier: | Total Height Safety |
| --- | --- |
| Type/Brand: | SafetyLink LadderLink Ladder Stabilising Dock |
| Use type: | Roof access |
| Finish/colour: | ~ |

4.11	SAFETYLINK FASTFIT LADDER SYSTEM

| Supplier: | Total Height Safety |
| --- | --- |
| Type/Brand: | SafetyLink FastFit Ladder System |
| Use type: | Roof access |
| Finish/colour: | ~ |

4.12	SAFETYLINK FASTFIT WALKWAY SYSTEM

| Supplier: | Total Height Safety |
| --- | --- |
| Type/Brand: | SafetyLink FastFit Walkway System |
| Use type: | Roof access |

4.13	SAFETYLINK FASTFIT GUARDRAIL SYSTEM

| Supplier: | Total Height Safety |
| --- | --- |
| Type/Brand: | SafetyLink FastFit Guardrail System |
| Use type: | Roof access |
| Finish/colour: | ~ |

4.14	SAFETYLINK FASTFIT HATCH GUARDRAIL SURROUND SYSTEM

| Supplier: | Total Height Safety |
| --- | --- |
| Type/Brand: | SafetyLink FastFit Hatch Guardrail Surround System |
| Use type: | Roof hatch access |
| Finish/colour: | ~ |
| 4.15 SAFETYLINK FASTFIT MODULAR STAIR SYSTEM Supplier:; Total Height Safety. Type/Brand:; SafetyLink FastFit Modular System. Use type:; Elevated level access. |  |